# ERIN GREENE & ASSOCIATES

Business Information Management Systems

2613 North 11th Street

Ocean Springs, Mississippi  39564-8364

Phone 228-875-6003

e-mail  rhartney@bellsouth.net

### SYSTEM OVERVIEW

AUGUST 2000

SYSTEM OVERVIEW

1.  THE PRODUCT

The objective is to produce a low cost, high performance, user operated

Lightpen/Mouse-controlled Display Terminal System.

The resulting system may be used in virtually any Commercial Small/Large

Business Application including but not limited to Federal/State/Local Government,

where present day usage is the Personal Computer (PC) with or without Network

Server Hardware/Software.  Simply discard the Tower/Server with whatever they

contain; retain the remaining devices; and plug in the  new Processing Hardware

and Application Programs.  This system is open-ended to all types of Peripheral devices.

A typical application could be ALL Departments and ALL associated functions in a

Hospital or GROUP of Hospitals.  Or it could be used by NASA as pre/post launch

Systems Monitor and Remote Control System. If you ask the question – CAN IT DO? –

The answer is YES.

The system is programmed in a language called TIPS.  TIPS is an acronym for

TOTAL INFORMATION PROGRAMMING SYSTEM.  TIPS is an interpretive language

which provides the Application Programmer with the capabilities to Collect, Store,

Manipulate, Structure and Retrieve blocks of data.  The language is therefore FILE, FORM, PAGE, and PART oriented.  TIPS has been specifically designed to function in a Multi-user, Multi-port environment & occupies only 80,000 bytes of Memory – more specifically –  9,967 Instructions.  The Application Programmer may create Forms at any User Station; enter related Programs; Test and Modify as needed, before or after the system is On-Line.  These features are not inherent in present day PC’s.  The system is designed to accommodate from 1 to 128 User Stations.  A parallel expansion port is provided for “N” sub-systems.

The USER STATION consists of a CRT/Flat Panel Monitor, Keyboard, Lightpen/Mouse, Printer, ID Reader, and a small assembly containing Interface Logic.  The assembly also provides connectors for the local components and one connection to a remote Computer Assembly.  A station may include/exclude the Printer and ID Reader.

The remote COMPUTER ASSEMBLY can accommodate up to 128 User Stations without noticeable degradation in response to user requests.  The mounting rack contains a Language Processor with required SRAM Memory resources and Input/Output Controllers;

and a Peripheral Processor with its required SRAM Memory resources and Input/Output

Controllers.  The two processors are identical ERIN32 FPGA devices having identical Instruction Sets.  Also provided is the necessary Power Supply, Cooling Fan Assembly, EMI

Line Filters, and connectors for shielded twisted pair cabling of Differential Driver/Receivers.

Fiber Optics may be used where distance is a factor (cost must be considered).

2.  HISTORY OF THE LANGUAGE

The language of TIPS is based upon JOSS (JOHNNIAC Open-Shop System which

was developed by the Rand Corporation in the early 1960’s.

Note:  Refer to: JOSS: A DESIGNER’S VIEW OF AN EXPERIMENTAL ON-LINE COMPUTER SYSTEM BY J.C. Shaw of the Rand Corporation, Santa Monica, California

PROCEEDINGS – FALL JOINT COMPUTER CONFERENCE, 1964.

Other efforts in similar developments were undertaken by the University of Pittsburg (PIL) – Pittsburg Interpretive Language) and Sanders Associates Incorporated (FOPS ) – File

Oriented Programming System.

3.  OLD SYSTEM ARCHITECTURE

The Sanders effort was started in 1965 and terminated in 1972. The system used two Honeywell minicomputers, a DDP-516 and a DDP-416.  The 416 had a reduced instruction set and but otherwise identical to the 516.  The total system was state-of-the art at the time.  Much effort was done in marketing – too no avail – due to cost – and the entire Department was disbanded.

System deficiencies included:

- Inadequate Core Memory for the Language Processor & therefore:

- User Slots limited to four (4) & therefore:

- Disk swapping was necessary & therefore:

- Disk access was 90 milli-seconds average & therefore:

- Six (6) or more Users of rapid Lightpen strikes resulted in noticeable

Slowing in response time.  Keyboard inputs were never a problem.

The sixteen (16) bit computer had limited addressing capability and forced Indirect or

Indexed Addressing for Memory Reference Instructions in too many instances.  Prior to being disbanded, an effort in redesign of the DDP-516 was pursued but not completed.

Disk storage was limited to eight (8) units which provided 6.4 Mbytes for Application Programs and Data.  A disk Sector contained 32 16-Bit Words, including overhead (pointers, descriptors).

The CRT Monitors were physically page-oriented capable of displaying 1019 characters

Including format types.  All cabling was direct twisted pair-parallel data.

4.  NEW SYSTEM ARCHITECTURE

This design is similar to its predecessor with two major exceptions.  The two processors are identical FPGA devices with similar ISA (Instruction Set Architecture) as the DDP-516.  This design ceased being an Emulation when 32 Bit Data was incorporated.  Several non-usable

Instructions were discarded and several instructions were added.  The purpose in retaining the ISA is to minimize changes to capture EXISTING SOFTWARE.  The other major difference is the use of SSRAM in lieu of Disk for Application Programs and Data Storage

The processor(s) Program Memory is Four-Port where:

Port 1 = Instruction Source

Port 2 = Vector (JMP, JST (CALL), JMPI (RTN), BRANCH, Interrupt (CALL)

This Port uses the Address Read-back of  the on-chip Program

Counter also having either Load (Vector) or Increment (PC+1) features.

AND Instruction Look-Ahead for the above Instructions

Port 3 = Boot Load

Port 4 = Reserved

There are three (3) SIMM/DIMM packages, each providing 64K X 36;

yielding an 108 Bit Micro Instruction.

The processor(s) Local Memory (Harvard Architecture) is Four-Port where:

Port 1 = Read Only (Operand Look-Ahead)

Port 2 = Read Only (Operand Look-ahead)

Port 3 = Write Only, Shared with Boot Load

Port 4 = FBC (Fully Buffered Channel)

From/To Global Memory &

From/To Associated Peripherals

Language processor Peripherals are CICU (Console Input Control Unit), CRT

Refresh Memory, Multiply/Divide/Square Root & Floating Point. Where the

Peripheral processor services up to 128 Printers, Real Time Clock, Fax, Modem,

up to 128 Page Scanners, Iomega Disk, and up to “N” Hard Disks.

There are four (4) SIMM/DIMM packages; each providing 64K X 36

and is allocated for each of the 128 USERS as follows:

- 4K Bytes – Display Memory

- 12K Bytes – Application Program and Local M2M Registers

The processor(s) Global Memory is Two-Port where:

Port 1 = FBC   From/To Language Processor Local Memory

Port 2 = FBC   From/To Peripheral Processor Local Memory

System Programmers will allocate an area of this memory for

Inter-processor mail-box.  An inter-processor Interrupt is provided

A System CRT Monitor with Keyboard is provided primarily for Debugging System

Software of either processor.  A Control Panel is also provided for Single – Stepping thru a

Program.  The processors are switch selectable. The Programmer can modify either Program Memory or Local Memory as necessary. The End-Item Hardware will NOT have this capability.

As a result – it is IMPOSSIBLE for VIRUS to enter the system.

5.  SYSTEM SOFTWARE

As previously stated; the Operating System consists of 9,967 Instructions including Hard

Disk operations for storage/retrieval of data and Application Programs.  This function is removed

from the Language Processor and allocated to the Peripheral side which includes Directory & File Maintenance for the Local system and up to “N” subsystems.  This reduces the instruction count to approximately 9,000.  This is FULLY Optimized Hand Coding using a Program Assembler – not a Compiler.  The system programmer is TOTALLY oblivious that the machine does or does not employ Pipeline design.  Once the System Programmer is fully satisfied with his design – this person, in association with a Micro-Programmer will further optimize the code using the full capabilities of the ERIN32 Processor.

To fully define the hardware requirements of the ERIN32; the following Instruction mix is used:

BRANCH		16%

STORE		  9%

ARITH/LOGIC	45%

LOAD			26%

SHIFT			  4%

An analysis of the 9,967 Instructions, which is a good basis for these percentages, shows that they are approximately correct.  A further analysis was accomplished, resulting in:

Reduces to:

JMP		1318			fully transparent

JST		  764			fully transparent

LDA/STA	  390 pairs		390

BRANCH	  500 pairs		500

LDA/ADD	  116 pairs		116

LDA/SUB	    86 pairs		  86

LDA/ANA	  148 pairs		148

LDA/ERA	    36 pairs		  36

LDA/AOA	    16 pairs		  16

LDA/CMA	      8 pairs		    8

LDA/TCA	    10 pairs		  10

LDA/CAS	    74 pairs		  74

LDA/IMA	    22 pairs		  22

LDA/LGR	    13 pairs		  13

LDA/ARR	      3 pairs		    3

LDA/LGL	    15 pairs		  15

LDA/ALS	      3 pairs		    3

LDA/ARS	      1 pair		    1

LDA/ALR	      2 pairs		    2

Total Reduction	1441

Total Inst after		7559

Other reductions can and will be made where 7500 is a good number for conversation.  Why concentrate in this area??  For future Research with possible ASIC application.

6.  THE PROCESSOR

A.  INTRODUCTION

The ERIN32 is a 32-bit binary data word general purpose computer with a

5 Nano-second Synchronous Static Ram (SSRAM) memory.  The machine organization

permits Direct, Index Relative, and Indirect addressing modes. This allows addressing

of 1,048,576 (20 bits) data or instruction words. Standard features include a flexible instruction repertoire of 67 commands, three hardware index registers (Harvard Architecture), 8 vectored interrupts and a powerful I/O bus structure which uses an FBC (Fully Buffered Channel) for data transfer.  A Symbolic Assembler is used to generate machine code having an 108 bit instruction word.  Options include high-speed arithmetic Multiply, Divide, Square Root and Floating Point, and a full line of peripheral equipment.

The ERIN32 is designed for both open-shop scientific applications and real-time

on-line data processing and control.  A broad variety of applications can be tailored to

include data reduction, process control, instrumentation, simulation, open-shop scientific

and engineering computation or any combination of these.

Programming the ERIN32 is similar to programming other dual source, single

destination address computers using two’s complement notation.  Therefore, no major differences confront the programmer who is new to the ERIN32.

B.  DESCRIPTION

The End-Item Processor is a Quicklogic Corporation QL6500 FPGA, 516 Pin device

of the Eclipse Family.  The development software for this family is currently in Beta Testing.

Its capability is:

Max Gates	 = 488,064

Logic Cells	 = 3072

Max FF’s 	 = 7488

Max I/O  	 = 448 pins

Ram Modules	 = 32 unused

Package	 = BGA

PLL		 = 4, with 1x,2x,4x

DLL

Global Clock Nets = 9

The primary reason for selecting this device is I/O pins where I require 395; not because I am logic limited.  In order to obtain a reasonably close view of the design, I removed the FBC logic and the associated pins and went thru PLACE & ROUTE using a QL3060-4 with the following results:

Usable PLD Gates	60,000

Logic Cells		1360 of 1584

Clock Only Cells	3 of 8

FF’s			876

Bi-Directional Cells	308 of 308

Routing Resources	39328 of 102916 (wires)

This is where delays are involved in addition to gate

delays as a function of LOADING.  All loading is held

by design to a maximum of 8. Here the term fan-out has

the same meaning.

ViaLink Resources	34327 of 2762290 (fuse links)

Delays that affect the design external to the device are based on these assumptions.  SSRAM access = 5NS, address set-up = 5ns from I/O pin to RAM pin.,  SSRAM data to input pin

= 5NS.  These are circuit board delays.

The Processing Elements (PE’s) and Execution Units (EU’s) include the first level of Pipeline where necessary including 32 bit Argument Registers.  Each contain their State Logic Timing, Scoreboard, Wait Logic for operations requiring results of previous instruction & a Multiplexer Steering output. In these designs you will see different methods of loading distribution techniques as described in the Quicklogic Manual.  Each PE has been FULLY hand logic optimized – it is further optimized in the CHIP generation process which modifies your requested function to that of the Logic Cell structure.  A print-out is available of the affected areas.  All design uses SCHEMATIC input ONLY.

AL32		This EU performs all of the required Arithmetic/Logic functions.  There is

no magic here – pure text book - TTL  - SN74181 ALU with the SN74182

used for Carry Look-Ahead.  Two of these EU’s are required for the Single Level Pipeline. It satisfies all of the functionality of the DDP-516 and will provide additional Instruction capability to the ERIN32.

AMUX	This PE is a simple multiplexer  that receives inputs from the AL32, the

Barrel Shifter (one input AS IS at the output, and is also bit reversed for

Right Shifts).  Two inputs are from the Load PE and one from the CSTB

EU.

AREG		This PE is a simple 32 Bit Register/Accumulator.  Data inputs are from the

AMUX;  Sign control and Clear functions are received from the UDRS (others) PE, such as PS (Preset Sign), RS (Reset Sign), Clear selected Byte, and CRA (Clear A-reg).  The AZ (Areg=Zero) output is essentially one bit of the CCR (Condition Code Register) used for Conditional Branch.  Every

instruction having Areg destination (which is most of the time) will cause AZ

To change.

BRAN	This EU contains a 16 Bit Instruction Register of the type of Branch the Programmer wants to execute.  He may select one or more Conditions. There is a 16 Bit OR gate whose output is BRAN. This output is applied to the VECTOR EU.

CAS	This EU performs an Arithmetic Compare of two operands and Registers

the result  in 3 bits of the CCR.  These are ALB, AGB, and EQU.  The

design uses the TTL SN74185 as building block.  Pure textbook design.

CSTB	This is a classical Data Selector/Multiplexer design that performs the Clear Bit, Set Bit and Test Bit Instructions and is based on TTL SN74150. The TST output is one bit of the CCR and is applied to the BRAN EU.  The Data Outputs are applied to the AMUX.

INDEX	The design uses three (3) of these EU’s.  They effectively implement the

Harvard Architecture chosen.  Here; the Programmer has many Options.

The A-Source and B-Source Index Register (with Increment capability)

are applied to SRAM Read Only Ports as previously described. This

source may be any combination of Common Data Pool and USER slot.

The Destination (Write) Index Register has the same variable available.

They may be randomly Incremented or Loaded one, two, or all three selectively.  The “X” output is applied to the Store EU.

IRS	Increment, Replace (Store) & Skip if Zero = DDP-516.  Increment, Replace & Register result = ERIN32.  This is one bit of the CCR and uses the term NZ.  This design uses a 32 Bit Incrementer where the INDEX EU uses a 20 bit Incrementer.  The NZ output is applied to the BRAN EU.  The Incremented value is applied to the STORE EU..  Two of these EU’s are required for the Single Level Pipeline that is implemented.

LOAD	This EU  receives Data from a Read Only Port and services the LDA

(Load Accumulator) and LDB (Load Selected Byte in Selected Byte

Position), with or without bit 8 INB (Inhibit) Instructions. Bit 8 is defined as the CURSOR BIT,  where Software is primarily interested in the remaining ASCII Character, INB is specified.  The only exception to this is the normal CRT Edit process.  Where typing says – enter the required character and Step the Cursor.  This can be done with the SETB (Set Bit) Instruction.

STORE	This EU receives selected inputs based on execution of  STA (Store

Accumulator), STB (Store Selected Byte in the Selected Byte Position, with or without INH), STX (Store Selected Index), IRS, ALU, or Shift.

The design currently uses a Single Level Data Pipeline; however; this will have to increase to properly match STORE requests (Queue) to the speed of the SSRAM.  The Store Address queue will have to increase a like amount.  A Logic ERROR if you like the term – I do.

UDRS	(Others)  This EU consists entirely of Instructions in the Control category.

Such  as  CRA,  SSM  (Set Sign Minus),  SSP (Set Sign Plus),  CSA (Complement Sign of Areg).  It contains the CFF which is one Bit of the CCR.  It may be controlled with SCB (Set C-bit), and RCB (Reset C-bit)

Instructions.  Other control of the CFF is the result of  Arithmetic and Shift Instructions. (Overflow/Underflow)

VECTOR	This EU receives JMP Unconditional Jump), JST (Jump and Store Return), BRA (Conditional Branch) and RTN (Jump Indirect) from Port 2 of Instruction Memory.  The ENB (Enable Interrupts), INH (Inhibit Interrupts) Instructions are received from Port 1.  Here I use the term Vector, where the next Instruction is PC+1 or go where Vectored.  At Honeywell we called this “Splatter”.  There is provision for eight (8) Vectored Interrupts, where level

8 is the highest level.  The highest Level is assigned to Inter-Processor Interrupt, then 7 is FBC Termination Interrupt.

The remainder are assigned to Peripheral Devices depending on the Processor application, i.e., Language or Peripheral.  These may change after discussing with Software guru’s.  A TTL SN74148 is used as the Priority Encoder.  Further timing analysis may require I Look-Ahead two rather than the current one. This is due to the clock to out of a JKFF, one four input gate and the transition time of Bi-Directional I/O Pad.  This is the TOTAL  Jump/Branch decision time.    I should note that the design does have a HLT (Halt) Instruction as does the DDP-516 – however the 516

virtually came to a HALT.  Currently; System Software is in a continuous loop in the Skedular Routine looking for something to do. This will change. The routine will make a single pass, and if Un-Interrupted will execute a HLT.  I have made provision for an Interrupt from HALT.  This also

applies to the Peripheral Processor. Interrupts must be ENABLED.

WADE	This EU is named for my Grandson Wade.  This was a fun design and

the little fellow is FUN.  IT is more appropriately called SHIFT.  The

design is based on the TTL AM25S10 (AMD), 74F350 (Fairchild) or

SN74S350 (TI).  All three sources provided excellent Application Data

Sheets.  The method I chose was to hard-wire a Shift Left.  Right Shifts

are Bit Reversed prior to application to the barrel shifter array.  This reversal is applied to the AMUX where it is reversed for the correct value.  The ENDL

(End Logic) provides the necessary input depending on the type of shift.

The FUN part was HOW to accomplish the ALS (Arithmetic Shift Left)

which is required to set the CFF for a change in Sign resulting from the Shift.  This is necessary to maintain Arithmetic correctness.  I chose to

do an un-registered Normalize (Shift Left till two MSB’s not Equal.).

This is accomplished by XOR of the Sign against each Bit;  apply this to a

Series of Priority Encoders (SN74148); encode the outputs of the 148’s and do a Logical Compare (SN74185) of the requested Shift amount with Encoded

Value – if Equal to or Less Than – clear CFF, if Greater – set CFF.  May

not be the only solution, but for an array shifter, I’m open for other design

approaches.  Two of these EU’s are required for a Single Level Pipeline

##### C.  INSTRUCTION SET

Memory Reference Instructions:		Addressing

JMP	Unconditional Jump			D, X, I

JST	Unconditional Jump			D, X

& Store Return

LDA	Load Accumulator			D, X

A-Source to Areg

STA	Store Accumulator			D, X

Areg to Destination

CAS	Arithmetic Compare			D, X

Compare A-Source to B-Source,

ALB, EQU, AGB into CCR

IRS	Increment				D, X

A-Source + 1 to Destination

LDX	Load Selected Index Register		D, X

A-Source to Xn

STX	Store Selected Index Register		D, X

Xn to Destination

LDB	Load Selected Byte of A-Source	D, X

in the Selected Byte Position

STB	Store Selected Byte in	the		D, X

Destination Selected Byte Position

IMA	Interchange Memory			D, X

A-Source & Accumulator,

Areg to Destination

ADD	Add A-Source + B-source		D, X

to Areg, Overflow into CFF

SUB	Subtract B-Source from A-Source,	D, X

To Areg, Underflow into CFF

ANA	Logical AND of A-Source		D, X

& B-Source to Areg

ERA	Logical Exclusive Or of A-Source	D, X

& B-Source to Areg

OR	Logical Or of A-Source		D, X

& B-Source to Areg

NOR	Logical Nor of A-Source		D, X

& B-Source to Areg

XNR	Logical Exclusive Nor of A-Source	D, X

& B-Source to Areg

NAN	Logical Nand of A-Source		D, X

& B-Source to Areg

ANB	Logical A and Not B of A-Source	D, X

& B-Source to Areg

NAB	Logical Not A and B of A-Source	D, X

& B-Source to Areg

###### OTHER ARITHMETIC

CMA	Complement A			NA

TCA	Two’s Complement A			NA

AOA	Add One to Areg, 			NA

Overflow into CFF

ACA	Add CFF to Areg,			NA

Overflow into CFF

AMO	Areg Minus 1,				NA

Underflow into CFF

Note:  All of the above Arithmetic and Logic & LDA instructions may include an optional Store.  For example: LDA/STA or ADD/STA.  In these cases a

Destination Address is specified.  The results will always be to the Areg

The A-Source is either the Areg or memory.

D = Direct

X = Index Relative

I = Indirect (Currently JMP only – this change to all Mem Ref Instructions)

SHIFT INSTRUCTIONS

LGL	Logical Left

The Areg or A-Source is shifted left (n) positions.

ZERO’s fill in vacated bit positions of Least Significant

End.  A32 is shifted to the CFF.  Result to Areg and

Destination if specified. (LGL/STA)

LGR	Logical Right

The Areg  or A-Source is shifted right (n) positions.

ZERO’s fill in vacated bit positions of Most Significant

End.  A1 is shifted to the CFF.  Result to Areg and

Destination if specified.  (LGR/STA}

ALR	Logical Left Rotate

The Areg or A-Source is shifted left, end-around (n) positions.

A32 is shifted to A1 and the CFF.  Result to Areg and

Destination if specified.  (ALR/STA)

ARR	Logical Right Rotate

The Areg or A-Source is shifted right, end-around (n) positions.

A1 is shifted to A32 and the CFF.  Result to Areg and

Destination if specified.  (ARR/STA)

ALS	Arithmetic Left Shift

The Areg or A-Source if shifted left (n) positions.

ZERO’s fill in vacated bit positions of Least Significant

End. If shifting causes a change in sign of Areg or A-Source

at any time during the instruction, the CFF is set.  If the sign

is not changed, the CFF is reset.  Result to Areg and

Destination if specified.  (ALS/STA)

ARS	Arithmetic Right Shift

The Areg or A-Source is shifted right (n) positions.

The sign bit (A32) does not change; it is shifted into vacated

positions of the Areg or A-Source (Sign Extended).  The CFF

takes the state of the last bit shifted out of the register.  Result

to Areg and Destination if specified.  (ARS/STA)

Note:  Shift amount (n), where (n) = 1 to 31

CONTROL INSTRUCTIONS

IRX	Increment Selected Index

SXS	Select A-Source Index

SXD	Select B-Source Index

SXW	Select Destination Index

CLB	Clear Bit (n)

Clear the selected bit

SBT	Set Bit (n)

Set the selected bit

TSB	Test Bit (n)

Test the selected bit

CBY	Clear Byte (n)

Clear the selected byte

Right Byte = 0

RCB	Reset CFF

SCB	Set CFF

CSA	Copy Sign and Set Sign Plus

A32 to CFF, Clear A32

(Have a logic error here)

CHS	Complement Areg Sign

CRA	Clear Areg

INH	Inhibit Interrupt

ENB	Enable Interrupt

HLT	Halt, or Wait for Interrupt if ENABLED with prior ENB

NOP	No Operation

#### BRANCH INSTRUCTIONS

BLT	Branch Less Than

Result of previous CAS

BEQ	Branch Equal

Result of previous CAS

BGT	Branch Greater Than

Result of previous CAS

BNZ	Branch Not Zero

Result of previous IRS (NZ)

TBZ	Test Bit Zero

TBNZ	Test  Bit Not Zero

SRC	Branch  if CFF reset

SSC	Branch if CFF set

SZE	Branch Areg Zero (AZ)

SNZ	Branch Areg Not Zero (AZ)

SLN	Branch A1 = One

SLZ	Branch A1 = Zero

SPL	Branch A32 = Zero (positive sign)

SMI	Branch A32 = One (negative sign)

SW1	Branch Switch 1 Set (ON)

An Application Programmer is Debugging,

DO NOT HALT ON ERRORS

SW2	Branch Switch 2 Set (ON),

Call System Programmer Debug Routine

BRA	 Branch

Note:	All of the above establish a condition that will be TESTED upon

Execution of BRA.  One OR more of these conditions may be

established prior to BRA.  For example; CAS establishes one of

three conditions.  I may execute BRA on Equal or Less Than if

BEQ/BLT is previously specified or may execute BRA/BEQ/BLT

in the same instruction.

INPUT/OUTPUT INSTRUCTION

Mov	Move A-Source data or one of the following controls

0 = Clear interrupt Level 1 (Lowest)

1 = Clear interrupt Level 2

2 = Clear interrupt Level 3

3 = Clear interrupt Level 4

4 = Clear interrupt Level 5

5 = Clear interrupt Level 6

6 = Clear interrupt Level 7

7 = Clear interrupt Level 8 (Highest)

8 = Send Inter-Processor Interrupt Pulse

9 = FBC Global Memory Write

A = FBC Port 4 Write

B = FBC Refresh Memory Write

C = CICU Read to FBC Port 4 Write (Language Processor),

Reserved (Peripheral Processor)

D = Reserved

E = Reserved

F = Reserved

Move A-Source Data from/to Global requires two MOV Instructions to setup Global Two-Port Address with Bank Select and Local FBC Port 4 Address, Bank Select and Range Count. Control field determines direction.

Move A-Source Data from/to Refresh Memory requires two MOV Instructions to setup Refresh Memory Two-Port Address and Local Port 4

Address, Bank Select and Range Count.  Control field determines direction.

OPTIONAL INSTRUCTIONS

The optional Multiply/Divide Processor requires an additional FPGA device.

Both Instructions are treated as asynchronous peripheral devices and may be initiated in any sequence – effectively executing in parallel. Both are initiated

with the Mov Instruction.

MULTIPLY – The design performs an Un-Signed 32 X 32 Time-Sequenced, pipelined multiply/add with Nibble Skip. (Nibble = 4 bits).  The multiplier must be applied first in sequence for examination of the data.  The result is a 64 Bit Product which requires two Mov instructions for inputting the MSP (Most Significant Product) and the LSP (Least Significant Product). Instruction completion is signaled with an Interrupt.  Information on Time

Sequence Multiply may be obtained from AMD Data Books for application of the AM25S05, a 2 X 4 Two’s Complement Multiplier.

DIVIDE – The design performs an Un-Signed 32 X 32 pipelined divide,

Skipping over strings of One’s and Zero’s.  The design is based on the text in

“The Logic of Computer Arithmetic: by Ivan Flores, Prentice-Hall, Inc., the

Library of Congress Catalog Card Number 63-14727.  The result is a 32 Bit

Quotient and 32 Bit Corrected Remainder.  The Divisor must be equal to or

greater than the Dividend.  The Divisor must be applied first for a

Normalization process to occur.  Outputting the Dividend will cause an

Equalization process to occur.  Two Mov instructions are required to first

input the Quotient and then the corrected remainder if desired.  Instruction

completion is signaled with an Interrupt.

These designs were completed but not tested in 1997 using a Quicklogic

QL3060-4.  The results of Place & Route are:

Logic Cells		          814 of 1584

Clock Cells		              2 of  8

Bi Directional Cells       	42 of 308

Routing Resources	      23213 of 102916

ViaLink Resources	      19951 of 2762290

Cell FF’s		          427 of 1584

This design will use the QL6250 of the Quicklogic Eclipse Series to obtain

the best possible performance.  Slight changes will be required to conform

to the Mov instruction and Interrupt Generation.

FLOATING POINT

Must first obtain the latest IEEE Specification.  Based on my experience in 1979-80 at the Honeywell Small/Medium Information Systems Division this design should take approximately two months.  Division WAS accomplished in the SAME manner as described above. Multiplication MAY be accomplished in the same manner as above however at Honeywell, this was

accomplished by examination of One’s and Zero strings as described by FLORES.  This design will use the QL6500 FPGA and should use the equivalent of 250 TTL MSI components.  The on-chip RAM will be used for storing Programs/Constants presented to it from FBC Port 4 of the Language Processor for execution.  I will assume the IEEE standard is 8 Bit Exponent and 24 Bit Mantissa and proceed accordingly.  It will conform to the Mov instruction and generate an interrupt on completion of a requested series.

GENERAL COMMENTS

You will notice I use a distributed processing scheme.  This will allow ALL

Processors the best possible Place & Route, fewest wires and best possible performance.